1.Draw a simple network diagram showing two routers connected over a WAN link and label where authentication is needed to prevent unauthorized access.

authentication is needed:

- Authentication verifies the identity of routers.

- Prevents unauthorized devices from connecting to the WAN.

- Protects network resources from unauthorized access.

2.   Configure a sample HDLC WAN connection between two routers using              Packet Tracer or any network simulator, and explain why HDLC does not provide authentication.<br><br><em><strong>Hint:</strong> Focus on the commands used and what is missing in terms of security.</em>

HDLC WAN Configuration Between Two Routers

Network Topology

Router1 ---------------- Router2
 S0/0/0                  S0/0/0
10.0.0.1/30            10.0.0.2/30

Router1 Configuration

enable
configure terminal

interface serial 0/0/0
 ip address 10.0.0.1 255.255.255.252
 encapsulation hdlc
 no shutdown

end
write memory

Router2 Configuration

enable
configure terminal

interface serial 0/0/0
 ip address 10.0.0.2 255.255.255.252
 encapsulation hdlc
 no shutdown

end
write memory

Verification Commands

Check whether the link is working: show ip interface brief

Test connectivity:

ping 10.0.0.2

(from Router1)

ping 10.0.0.1

(from Router2)

If the pings are successful, the HDLC WAN connection is working correctly.

Why HDLC Does Not Provide Authentication

HDLC (High-Level Data Link Control) is a Layer 2 WAN encapsulation protocol used to transmit data between routers. While it efficiently encapsulates data, it does not include any authentication mechanism.

Notice the configuration commands:

encapsulation hdlc

There are no commands for usernames, passwords, or authentication protocols. Any router configured with compatible HDLC settings can establish a connection on the link.

Missing Security Features

- No username verification

- No password verification

- No identity checking of connected devices

- No protection against unauthorized routers joining the WAN link

Because of these limitations, HDLC is considered less secure than protocols such as PPP (Point-to-Point Protocol), which supports authentication methods like:

- PAP (Password Authentication Protocol)

- CHAP (Challenge Handshake Authentication Protocol)

3.  Set up a PPP connection between two routers in Packet Tracer and enable PAP (Password Authentication Protocol) authentication. Show the configuration commands and explain what happens if the password is incorrect.

PPP Connection with PAP Authentication Between Two Routers

Network Topology

Router1 ---------------- Router2
 S0/0/0                  S0/0/0
10.0.0.1/30            10.0.0.2/30

PPP + PAP Authentication

Router1 Configuration

enable
configure terminal

hostname Router1
username Router2 password cisco123
interface serial 0/0/0
 ip address 10.0.0.1 255.255.255.252
 encapsulation ppp
 ppp authentication pap
 no shutdown
exit

Router2 Configuration

enable
configure terminal
hostname Router2
username Router1 password cisco123
interface serial 0/0/0
 ip address 10.0.0.2 255.255.255.252
 encapsulation ppp
 ppp pap sent-username Router2 password cisco123
 no shutdown
exit

Verification Commands

Check PPP status: show interfaces serial 0/0/0

Check authentication: show running-config

Test connectivity: ping 10.0.0.2

Expected Result:

Success rate is 100 percent (5/5)

How PAP Authentication Works

- Router2 sends its username and password to Router1.

- Router1 compares the received credentials with its local username database.

- If the credentials match, authentication succeeds.

- The PPP link becomes active and data transmission begins.

What Happens If the Password Is Incorrect?

Suppose Router2 is configured with:

ppp pap sent-username Router2 password wrongpass

while Router1 expects:

username Router2 password cisco123

Result

- Authentication fails.

- PPP session is not established.

- Routers cannot exchange data.

- Ping requests fail.

- Serial interface may show:

4.Modify your PPP configuration to use CHAP (Challenge Handshake Authentication Protocol) instead of PAP, and describe one key difference in how CHAP improves security over PAP.

To transition the point-to-point link from PAP to CHAP (Challenge Handshake Authentication Protocol), we must configure local usernames matching the hostname of the remote router and switch the authentication command on the interface.

Router1 Configuration

Plaintext

Router1> enable

Router1# configure terminal

Router1(config)# hostname Router1

Router1(config)# username Router2 password cisco123

Router1(config)# interface serial 0/0/0

Router1(config-if)# ip address 10.0.0.1 255.255.255.252

Router1(config-if)# encapsulation ppp

Router1(config-if)# ppp authentication chap

Router1(config-if)# no shutdown

Router1(config-if)# end

Router1# copy running-config startup-config

Router2 Configuration

Plaintext

Router2> enable

Router2# configure terminal

Router2(config)# hostname Router2

Router2(config)# username Router1 password cisco123

Router2(config)# interface serial 0/0/0

Router2(config-if)# ip address 10.0.0.2 255.255.255.252

Router2(config-if)# encapsulation ppp

Router2(config-if)# ppp authentication chap

Router2(config-if)# no shutdown

Router2(config-if)# end

Router2# copy running-config startup-config

Verification Commands

To verify that the configuration is working successfully and the link is up, use the following troubleshooting commands:

- show interfaces serial 0/0/0: Check that the line protocol is up (e.g., Serial0/0/0 is up, line protocol is up (encapsulation PPP)).

- show running-config: Verify that ppp authentication chap is active and passwords match.

- ping 10.0.0.2 (from Router1): Confirms end-to-end layer 3 connectivity over the authenticated link.

Key Security Difference: CHAP vs. PAP

| Feature | PAP (Password Authentication Protocol) | CHAP (Challenge Handshake Authentication Protocol) |
| --- | --- | --- |
| Password Transmission | Sends the password in plain text across the link. | Never sends the password over the network. |
| Mechanism | Simple two-way handshake (Username/Password sent, Accept/Reject returned). | Three-way Challenge-Response handshake using an MD5 hashed value. |
| Frequency | Authenticates only once during the initial link establishment. | Repeatedly re-authenticates the remote device at random intervals during the session. |

5.Research and list two real-world scenarios (such as public WiFi login or mobile data authentication) where WAN authentication protocols like PAP, CHAP, or MS-CHAP are used, and explain why authentication is critical in each case.

- Public WiFi Login (Hotels, Airports, Cafes)

When users connect to a public WiFi network, authentication is often required before internet access is granted. Protocols such as PAP, CHAP, or MS-CHAP may be used by the network equipment to verify the user's credentials.

Why Authentication Is Critical:

- Prevents unauthorized users from accessing the network.

- Protects network resources and bandwidth.

- Helps track user activity for security and management purposes.

- Reduces the risk of cyberattacks from unknown devices.

- Mobile Data Authentication (4G/5G Networks)

Mobile service providers authenticate smartphones before allowing them to access mobile data services. Authentication protocols verify that the user has a valid SIM card and subscription.

Why Authentication Is Critical:

- Ensures that only authorized subscribers can use the network.

- Prevents fraud and unauthorized access.

- Protects customer data and billing information.

- Maintains the security and reliability of the mobile network.